Palo Alto OS Upgrade step by step Process

CRQ00000515633 – Upgrade GSIL Firewalls to PAN-OS 11.1.4-h18

Log into Citrix

- QA - https://workspaceqa.sce.com/Citrix/SCEGridStore_QAWeb/

- PROD - https://workspace.sce.com/logon/LogonPoint/tmindex.html

- GSIL -

Log into PAN

- QA

- PROD

- GSIL

Log into Spectrum using Citrix / .26

- https://oneclick.gnscet.com:8443/spectrum/admin/index.jsp

- Prod / https://alhxvprcasoc03:8443/spectrum/webapp

- Prod : https://oneclick.gnsce.com:8443/spectrum/

- ID: gutiers@gnsce

- ID: gutiers@gnscet

- PW – Pin / Token

One Note

- PAN OS Upgrades

Palo Alto calendar

- https://intranetsps.prod.edisonintl.com/sites/it/gridcsn/Change%20Management/NSO%20Reference/PAN-OS%2011.1.4-h18%20Upgrade/11.1.4-h18%20Schedule%20Summary.xlsx?d=w388921f2951f4c9b86986e788633d762

Sharepoint

- 2023 - All Documents (edisonintl.com)

GSIL: Grid Services Integrated Lab

Before outage / Prep

- Evidence

- Checksum

- Store both in NERC Sharepoint Folder

To Download versions from Pan (Can be processed before Date of outage)

- Panorama

- Left hand panel / Device Deployment / Software

- Find version you need

  - Make sure of device model ex: PA-3260 will need PanOS_3200 versions

  - If you don’t see version click on Check Now

- Click on Validate

  - If validate is not an option / Click on download. Once download is complete / Validate option will now appear

- This will bring up new window ( Required Deployment ) / Type in device / click on deploy / click on download

During download / Log into Secure CRT to monitor progress

Type in / show jobs all

Type in / request system software install version – This will tell you what version we have on device

Make sure to update One Note with all times of actions

- GSIL Lab

- MOP:

- Capture Pre-Evidence

- CIP-10 Software Integrity Check Sum

CA-Spectrum Maintenance Mode

Id : gutiers@gnscet.com

Pw: Pin/RSA

- Click on device in left panel / Information tab/ In maintenance / Click on set

- Will bring up pull down menu / Click on Yes

- Status will be yes and Device will turn yellow

- Do to both devices in set ( Active / Passive )

Install Process

Process Passive side first

Backup FW Configurations

In device PA

- Device/ show jobs all

- Setup / Operations / Save: Save named configuration snapshot (Click on ) / Will bring up Named Configuration box – click on pull down, find device and put in current date / click ok

- Click on close

- Click on Export: Export named configuration snapshot / Will bring up Export Named Configuration box – click on pulldown, find device and date / click on OK

- Click on Export: Export device state

CRI Output

- Secure CRT / Log into device

- Call GSOC

- Send Email Notification

Start outage

Find out which to do first / You want to do Passive side 1st

- Log into each device using IP Address

- Make a note of ports .

- Spectrum

  - Put in Maintenance

- To find active in PA dashboard/high availability – Local (Active)

- Process Passive firewall first

- Suspend Passive firewall

  - Device / High Availability / click on Suspend local device for High Availability

- Click on OK

- Verify device is in suspend mode Dashboard / High Availability

- Verify active device Peer should now read Suspended

- Make a not of ports affected to verify they come back up after outage

- Ports being used will be green. Check after reboot to verify same ones are up.

- Check to make sure traffic is still flowing

To start outage / Install

- Check to see if device has version PAN-OS 10.2.6

- Check to see what version is on device . Dashboard / General Information

- Install PAN-OS 11.1.4-h18

- Left panel Device / Software / look for 11.1.4-h18 / Click on install

- Click on OK

- You can see Status

- Open up Secure CRT

- Type / Show jobs all

- You are looking at the type area / SWInstall

- When completed you will receive Reboot request

- Before you start reboot go back to Secure CRT

- Go Back to Secure CRT / check status of SWInstall / Should be FIN / OK

- Click on Yes to start reboot

- To find status / go to active look at Dashboard / High Availability / click on refresh/ It will turn yellow. You will know when Reboot completes. On opposite device all areas will turn green except for mismatches (Which s to be expected)

- Once it comes up / Log into device again / checking for new version

- To verify everything came up / Reconnect to  Secure CRT

- Show clock / show system info / Show jobs all

- Show jobs all – look for Autocom – It should show Status : FIN / Result – OK

- Verify ports are up. Should be same ones as before outage

- Pull up info

- Mismatches are expected. Active will have a different version

Active Firewall Install

- Go to active firewall / Device / High Availability / Operational Commands

- Click on Suspend local device for High Availability

- Dashboard – Should now read / Local is Suspended/ Peer is Active

- Double check that Active one is picking up traffic

- Monitor / Traffic – look at receive time – You might have to click on refresh

- Now you can start Install on 2nd device

- Same process as 1st

- The lowest device is usually the Active

- Open up Secure CRT

  - Show clock / set cli pager off / show system info / show cli info

- Setup / Operations / Save: Save named configuration snapshot (Click on ) / Will bring up Named Configuration box – click on pull down, find device and put in current date / click ok

- Click on close

- Click on Export: Export named configuration snapshot / Will bring up Export Named Configuration box – click on pulldown, find device and date / click on OK

- Click on Export: Export device state

CRI Output

- Secure CRT / Log into device

- Suspend firewall

  - Device / High Availability / click on Suspend local device for High Availability

- Click on OK

- Verify device is in suspend mode Dashboard / High Availability

- Verify device should now read Suspended

- Make a note of ports affected to verify they come back up after outage

- Ports being used will be green. Check after reboot to verify same ones are up.

To start outage / Install

- Check to see if device has version PAN-OS 10.2.6

- Check to see what version is on device . Dashboard / General Information

- Install PAN-OS 11.1.4-h18

- Left panel Device / Software / look for 11.1.4-h18 / Click on install

- Click on OK

- You can see Status

- Open up Secure CRT

- Type / Show jobs all

- You are looking at the type area / SWInstall

- When completed you will receive Reboot request

- Go Back to Secure CRT / check status of SWInstall / Should be FIN / OK

- To find status / go to active look at Dashboard / High Availability / click on refresh/ It will turn yellow

- Once it comes up / Log into device again / checking for new version

- To verify everything came up / Reconnect to  Secure CRT

- Show clock / show system info / Show jobs all

- Show jobs all – look for Autocom – It should show Status : FIN / Result – OK

- Verify ports are up. Should be same ones as before outage

- Pull up info

- Device will have a different version

To Make FW01 the active FW / Dashboard look at Dashboard on Active device EX: fw02 / High availability  , if FW02 is active , Change to Passive state

On Active device (EX FW02) Device / High Availability/ Operational Commands/ Click on Suspend local device for High availability

Click on OK

This will change display / Click on Make local device functional for High Availability

Now Dashboard will read that FW02 is in Passive state / FW01 is now in Active state

Confirm traffic is passing on active device

To Close out CRQ

Open Helix / Remedy / More – Release Management

- This will open up

Log into device

Issue show commands:

- Show clock

- Set cli pager off

- Show system info

- Show cli info

- Show jobs all

Debug Command needs to be issued on PA 850 devices after completion of upgrade